FIEC  -  Licenciatura en Redes y Sistemas Operativos

Fundamentos de Redes Inalámbricas

Examen Parcial	Diciembre 07, 2010

Profesor: Ing. Néstor Arreaga  A.	Alumno:

1.-Consteste lo siguiente

a.- Describa los conceptos de los siguientes términos: EIRP________________________________________________________________________ Beacons ______________________________________________________________________ CAM ________________________________________________________ Warchalk_________________________________________________

b.- ¿Cuál es la diferencia entre la unidad de medidas en decibelios dbm, dbi y dbd?
__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

c.- Describa cuales son los beneficios que ofrecen las WLAN.

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

d.- Describa Brevemente las características de las siguientes tecnologías:

802.11i _______________________________________________________________________

802.11e _____________________________________________________________________

802.11.f ____________________________________________________________________

e.- Indique y describa Los fenómenos que afecta a las RF en la transmisión de datos

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

f.- Describa todas las características del proceso de Roaming que realizan los AP en las redes inalámbricas.

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

g.- Que opciones de configuración ud. debe establecer en un AP Aironet  1200 para establecer una pequeña red inalámbrica funcional y que entregue apropiadamente las direcciones ip a los clientes . Así mismo diseñe un grafico que defina la topología de la red que va a elaborar, use CLI para su configuración.

2.- Answer the question

a.-What causes the Status LED to blink green in a 1200 series AP?

transmitting/receiving packets

no devices associated

general warning

no special meaning

b.- When configuring the AP Radio Port Hardware setup page, which settings are recommended in situations where communication is poor and many client devices are associating with the access point?

low fragment threshold and high RTS threshold

high fragment threshold and high RTS threshold

high fragment threshold and low RTS threshold

low fragment threshold and low RTS threshold

c.- Which command is used to set the parent AP for a repeater AP?

AP(config)# parent 10.0.0.1

AP(config-if)# parent 1 10.0.0.1

AP(config-if)# parent 1 abcd.1234.ab12

AP(config)# parent abcd.1234.ab12

d.- What is the behavior of a wireless bridge when configured as a root bridge?

It associates and communicates with any wireless device on the network.

It associates and communicates only with other root bridges.

It associates and communicates with multiple non-root bridges but only a single root bridge.

It associates and communicates with multiple non-root bridges but will not communicate with other root bridges.

f.- On the Cisco AP 1200 series, what is the name of the 802.11a radio interface?

interface dot11radio 0

interface dot11radio 1

interface dot11radio a

interface 802.11a radio

3.- Defina las características de básicas y fundamentales para implementar redes inalámbricas usando Access Point y Brigde.